Wind Power Plants

逐页英文演讲稿与中文提示

正文对应 PPT 第 1 至 20 页。讲者 A 负责第 1 至 11 页，讲者 B 负责第 12 至 20 页，各约 10 分钟。英文用于正式口播，中文用于理解和排练。

英文正文约 2,224 词。20 分钟包含指图、解释数字和换页停顿，以下时间为排练目标。第 21 至 23 页是参考文献，第 24 至 26 页是问答备份，PPT 已将这些页设为隐藏。

页序与时间

| 页 | 讲者 | 时间 | 知识点 |
| --- | --- | --- | --- |
| 1 | A | 00:00–00:30 | Wind Power Plants |
| 2 | A | 00:30–01:15 | Wind as an energy source |
| 3 | A | 01:15–02:15 | How blades turn the rotor |
| 4 | A | 02:15–03:20 | The generator inside the nacelle |
| 5 | A | 03:20–04:20 | Collecting electricity from a wind farm |
| 6 | A | 04:20–05:05 | Yaw control |
| 7 | A | 05:05–05:55 | Pitch control |
| 8 | A | 05:55–07:00 | Wind speed and available power |
| 9 | A | 07:00–08:05 | A wind-power calculation |
| 10 | A | 08:05–08:55 | The power coefficient Cp |
| 11 | A | 08:55–10:00 | Capacity factor and annual energy |
| 12 | B | 10:00–11:25 | Frequency support from active power |
| 13 | B | 11:25–12:40 | Voltage support from reactive power |
| 14 | B | 12:40–13:55 | Life-cycle carbon emissions |
| 15 | B | 13:55–15:00 | Siting and local impacts |
| 16 | B | 15:00–15:55 | Global wind capacity |
| 17 | B | 15:55–16:55 | China’s resource and demand locations |
| 18 | B | 16:55–18:15 | Hornsea 2 offshore wind farm |
| 19 | B | 18:15–19:20 | Wind in the generation mix |
| 20 | B | 19:20–20:00 | The output of a wind power plant |

Slide 1  Wind Power Plants

讲者 A    00:00–00:30    30 秒    风力发电

English speaking script

Good afternoon, everyone. Today we are going to explain how a wind power plant works. We will start with the turbine, then follow the electricity to the grid. I will cover the equipment and the main performance measures. My partner will explain grid support, environmental impacts, and a real offshore wind farm.

中文理解与指图提示

用正常语速介绍主题与分工。不要在开场把后面所有知识点都念一遍。

Slide 2  Wind as an energy source

讲者 A    00:30–01:15    45 秒    风的动能

English speaking script

The energy comes from moving air. We can see a rotor placed high above the ground, where it can intercept that air flow. The turbine does not burn a fuel to generate electricity. But it also cannot choose when the wind will blow. Even at a good site, wind speed changes during the day and across the seasons. That is why a turbine's rated power alone cannot tell us how much electricity it will produce. We need to understand both the machine and the wind at its location.

中文理解与指图提示

指着照片中的转子和塔架，解释能源来自空气运动。最后一句引出后面的功率与容量因子，不展开环境影响。

Slide 3  How blades turn the rotor

讲者 A    01:15–02:15    60 秒    升力怎样带动转子

English speaking script

Modern wind turbine blades work in a similar way to airplane wings. If we look at the cross-section, we can see that it has a special shape called an airfoil.

When the wind flows around the blade, it creates lift and drag. Part of this force helps the blade rotate, creating torque and turning the rotor.

Here, we can see that the blades are connected to the hub. They rotate together and transfer mechanical power to the turbine.

So, the important thing is that the wind doesn't just push the blades directly. Instead, the aerodynamic force helps the rotor spin.

Then, the generator uses this rotation to produce electricity. We will talk about that on the next slide.

中文理解与指图提示

指叶片、轮毂，按“气流产生气动力，切向分量产生转矩”解释。插图只说明转子位置，不是升力矢量图。

Slide 4  The generator inside the nacelle

讲者 A    02:15–03:20    65 秒    机舱中的发电机

English speaking script

The part behind the rotor is called the nacelle. In this picture, we can see a direct-drive design. This means the rotor is connected directly to the generator, so there is no gearbox.

The generator changes mechanical energy into electrical energy using electromagnetic action.

Another design uses a gearbox between the rotor and the generator. The gearbox increases the rotational speed, so it can use a different type of generator.

Both designs can generate electricity, but their structures and maintenance needs are different.

The tower supports the nacelle and lifts the rotor higher to catch the wind, while the foundation supports the whole structure.

So, the main thing to remember here is where the generator is located and how the rotor drives it.

中文理解与指图提示

图是直驱结构，先指 Direct Drive Generator，再说明有齿轮箱机型是另一种设计。不要把图讲成所有风机统一结构。

Slide 5  Collecting electricity from a wind farm

讲者 A    03:20–04:20    60 秒    风场集电与并网

English speaking script

One wind turbine is one generating unit. A wind farm connects many turbines together through a collection network.

In this picture, we can follow the cables from the turbines to the substation. The electricity from all the turbines comes together there. Then, transformers increase the voltage so the electricity can connect to the power grid.

Why do we need higher voltage? Because for the same amount of power, higher voltage means lower current. This helps reduce power losses during transmission.

A wind farm also needs other equipment, such as protection devices, meters, switches, and communication systems.

So, the main idea is that the collection network connects many individual turbines and sends their electricity to the grid through one shared connection.

Offshore wind farms work in a similar way, but their cables need to withstand the marine environment.

中文理解与指图提示

指图中的电缆、变电站与输电线路。讲清集电系统如何连接多台风机，不再复述机舱内部组成。

Slide 6  Yaw control

讲者 A    04:20–05:05    45 秒    偏航控制

English speaking script

Wind direction can change, so yaw control helps the turbine face the wind.

A wind vane measures the wind direction, and yaw motors turn the nacelle at the top of the tower.

As we can see, the whole nacelle turns with the rotor. This helps the turbine capture more wind energy and reduces uneven loading.

So, yaw control turns the whole nacelle, not the individual blades.

中文理解与指图提示

指机舱与塔顶之间的偏航机构。用“整台机舱转向”解释，避免把偏航与变桨混为一谈。

Slide 7  Pitch control

讲者 A    05:05–05:55    50 秒    变桨控制

English speaking script

Pitch control changes the angle of each blade where it connects to the hub. This changes the aerodynamic force on the blade.

When the wind is strong, the controller adjusts the blade angle to keep power and loads within safe limits.

If the turbine needs to stop, the blades turn to a feathered position to reduce the force from the wind.

So, more wind does not always mean more electricity. The main point here is that pitch control adjusts the blade angle to control power and keep the turbine safe.

中文理解与指图提示

指叶根，说明变桨改变叶片绕自身长轴的角度。重点解释额定功率以上的限功率与保护作用。

Slide 8  Wind speed and available power

讲者 A    05:55–07:00    65 秒    风速的立方关系

English speaking script

This equation shows why wind speed is so important. Wind power depends on air density, rotor area, and wind speed cubed.

The key point is the cubic relationship. If wind speed increases from 8 to 10 meters per second, the available wind power becomes about 1.95 times higher. At 12 meters per second, it becomes about 3.38 times higher.

However, a real turbine cannot always produce more power as wind speed increases, because the controller limits its output to the rated power.

中文理解与指图提示

先讲 v³，再指三个柱子。强调图表示来流可用功率比，不是某台风机的实测功率曲线。

Slide 9  A wind-power calculation

讲者 A    07:00–08:05    65 秒    风电功率算例

English speaking script

Let's look at a simple example. Assume the rotor diameter is 120 meters, the wind speed is 8 meters per second, and the air density is 1.225 kilograms per cubic meter.

Using the equation, the power in the wind is about 3.55 megawatts. With a power coefficient of 0.45, the rotor captures about 1.6 megawatts.

After accounting for 94% efficiency, the electrical output is about 1.5 megawatts.

This is just an example calculation, not actual measured data.

中文理解与指图提示

按来流 3.55 MW、转子 1.60 MW、电气输出 1.50 MW 依次指读。尺寸、风速、Cp、η 都是假设值。

Slide 10  The power coefficient Cp

讲者 A    08:05–08:55    50 秒    风能利用系数

English speaking script

The power coefficient, or Cp, tells us how much wind power the rotor converts into mechanical power.

But why is there a limit? Because air still needs to flow through and leave the rotor. In theory, the maximum Cp is about 59.3%, according to the Betz limit.

In real turbines, aerodynamic losses and equipment losses reduce the final output even more.

So, Cp represents how much power the rotor captures, while efficiency, or eta, represents how well the remaining system converts that power into electricity.

中文理解与指图提示

口播把 Cp 读作 C p。说明 Betz 上限是理想模型里的气动捕获上限，不能把它直接当整机电气效率。

Slide 11  Capacity factor and annual energy

讲者 A    08:55–10:00    65 秒    容量因子与年发电量

English speaking script

Capacity factor tells us how much energy a turbine produces over time compared with its maximum possible output.

For example, a 3-megawatt turbine with a 35% annual capacity factor produces about 9.2 gigawatt-hours of electricity per year.

The 35% does not mean the turbine is only 35% efficient. It depends on wind conditions, maintenance, and other losses.

That’s all for the turbine and performance section. Now, I’ll hand over to my partner to explain the grid connection.

中文理解与指图提示

按分子和分母解释容量因子。算例用非闰年的 8,760 小时。结尾完成 A 到 B 的交接。

Slide 12  Frequency support from active power

讲者 B    10:00–11:25    85 秒    有功功率与频率支持

English speaking script

Thank you. I'll start with frequency. When we don't generate enough electricity to meet demand, frequency tends to drop. A wind farm with suitable controls can help by increasing its active power.

But where does that extra energy come from? For a quick response, the turbine can use some of the energy stored in its spinning rotor. That won't last forever, and the rotor needs to build that energy back up afterward.

For a longer response, we need enough wind and some room to increase output. Look at this example: two megawatts are available, but we're only producing one point six. That leaves zero point four megawatts of headroom. If we're already using all the available wind power, we can't keep increasing output without another energy source.

中文理解与指图提示

指例子中的 1.6 MW 和剩余 0.4 MW。区分转子短时动能释放与持续有功备用，例子不是实测运行记录。

Slide 13  Voltage support from reactive power

讲者 B    11:25–12:40    75 秒    无功功率与电压支持

English speaking script

Next, let's look at voltage. Here, reactive power is important. In our full-converter example, the converter adjusts reactive current to help support the grid voltage.

The turbines also need to work together. A plant controller coordinates them so that the whole wind farm follows the voltage or reactive-power target at its grid connection.

Of course, there are limits. The converter can only handle so much current and heat. Active and reactive power both have to stay within those limits.

This links to our voltage-control lectures. A conventional synchronous generator uses an automatic voltage regulator to adjust excitation. This turbine uses converter control instead. The picture shows the grid connection, rather than the control circuit. Active power mainly helps with frequency, while reactive power helps with voltage.

中文理解与指图提示

重点讲变流器无功电流与风场协调。配图用于指出并网位置，不当作控制回路图。同步机 AVR 的详细比较留在备份页。

Slide 14  Life-cycle carbon emissions

讲者 B    12:40–13:55    75 秒    全生命周期碳排放

English speaking script

Now, what about carbon emissions? Wind turbines don't burn fuel when they generate electricity. But making, installing and maintaining them still causes emissions. We need materials like steel and concrete, so we have to look at the whole life cycle.

That's what this chart does. It uses median figures from the IPCC's 2014 assessment. For onshore wind, the figure is about eleven grams of carbon-dioxide equivalent per kilowatt-hour. For gas combined cycle, it's four hundred and ninety. For pulverized coal, it's eight hundred and twenty, in the same unit.

Looking at the bars, wind's emissions are much lower, but they're not zero. These figures bring together earlier studies. They're not measurements of every power plant, and they're not current-year averages. They give us a useful comparison. We also need to think about local impacts, which I'll cover next.

中文理解与指图提示

指三根柱子，明确单位和 2014 文献中位数口径。不要称为零排放，也不要把它说成 2026 年实时数据。

Slide 15  Siting and local impacts

讲者 B    13:55–15:00    65 秒    选址与周边环境

English speaking script

So, where should we build a wind farm? We need to consider birds, bats, habitats and nearby residents, as well as carbon emissions.

Surveys help us find sensitive areas, and monitoring tells us when we need to change how the turbines operate. Sometimes, temporarily reducing output or stopping turbines can lower the risk to wildlife. That's called curtailment. Noise and the way turbines look also matter to local communities.

Now look at the spaces between the turbines in this photo. The whole wind-farm area isn't all covered by concrete. Foundations and roads take up only part of it. Depending on the site, farming or other activities can continue between turbines. So we should separate the total project area from the land directly occupied by the equipment.

中文理解与指图提示

用照片解释风场总范围与基础、道路的直接占地不同。当地生态、噪声和视觉影响都属于选址问题。

Slide 16  Global wind capacity

讲者 B    15:00–15:55    55 秒    全球风电装机

English speaking script

Let's step back and look at the global picture. According to IRENA's 2025 report, the world had about one thousand one hundred and thirty-three gigawatts of installed wind capacity at the end of 2024. That's roughly one point one three terawatts.

China has the largest national installed-capacity market. The United States and Europe also have large wind fleets. Different regions build onshore or offshore projects depending on their wind resources and grid connections.

One thing to remember: gigawatts measure rated power. This tells us the size of the installed equipment, not how much electricity it produces in a year. For annual generation, we'd use an energy unit, like terawatt-hours.

中文理解与指图提示

数据明确写到 2024 年底，沿用原稿年份。把 GW 的额定功率含义讲清即可，不把它当发电量。

Slide 17  China’s resource and demand locations

讲者 B    15:55–16:55    60 秒    中国的资源与负荷分布

English speaking script

China shows why location matters. Strong onshore wind resources are often in the north and west, while major demand centres are elsewhere, including the east.

So, even with great wind, a farm may need long transmission lines to reach consumers. If the grid can't take all the available power, the farm may have to reduce its output. That's called curtailment.

Forecasting helps operators prepare for changes. Transmission and a flexible power system help them use the energy. Offshore projects can be closer to coastal demand, but building and maintaining equipment at sea brings extra challenges. Good wind is only part of the story: the electricity has to reach the people who need it.

中文理解与指图提示

用左右两栏说明资源所在地与负荷中心的关系。英国海缆照片不拿来冒充中国工程；本页用可编辑对照表。

Slide 18  Hornsea 2 offshore wind farm

讲者 B    16:55–18:15    80 秒    Hornsea 2 海上风场

English speaking script

Let's look at a real project: Hornsea Two. It's an offshore wind farm in the North Sea, off the UK coast, and it became fully operational in August 2022.

It has one hundred and sixty-five turbines, each rated at eight megawatts. Multiply those numbers and we get one thousand three hundred and twenty megawatts, or about one point three gigawatts.

The photo shows the turbine and the offshore setting, and the map shows where the project sits off the coast. The turbines need a collection network, substations and export cables to bring their electricity ashore. Maintenance also needs vessels and suitable weather conditions.

This brings our earlier topics together. Each turbine converts wind into electricity, and the shared electrical system delivers that electricity to the grid. Just remember: one point three gigawatts is the rated capacity. It doesn't mean the wind farm produces that much power all the time.

中文理解与指图提示

先指照片，再指右侧官方位置图。只保留已核对的投运时间、165 台与 8 MW，不使用“目前全球最大”。

Slide 19  Wind in the generation mix

讲者 B    18:15–19:20    65 秒    风电与其他电源的配合

English speaking script

How does wind fit alongside other power plants? Let's look at this table. Wind provides low-carbon electricity without buying fuel, but its output changes with the wind.

Gas plants can adjust their output within operating limits, but they need fuel and produce emissions. Solar PV depends on daylight and weather. Reservoir hydro can adjust output when water and operating conditions allow, but suitable sites and water availability are limited, and environmental impacts matter too.

The grid has to meet demand every hour, not just produce enough energy over a year. Flexible generation, connections to other grids, demand response and sometimes storage help manage wind's changing output. So these technologies need to work together.

中文理解与指图提示

表格只比较供电特性和主要限制。水电特指水库式，避免暗示所有水电都可自由调度。

Slide 20  The output of a wind power plant

讲者 B    19:20–20:00    40 秒    风电输出需要怎样判断

English speaking script

So, here's the main point we'd like you to remember: rated capacity tells us how big a wind farm is, but it doesn't tell us how much electricity it actually delivers.

For that, we need to look at the wind, the energy produced over time, and the grid connection. That's why our annual-energy example and Hornsea Two's capacity figure describe different things.

Thank you for listening. We're happy to take your questions.

中文理解与指图提示

只收束到“额定容量与实际交付不同”这一点。说完邀请提问，再按需要打开备份页。

问答准备

Why can a turbine not capture all the wind energy?

讲者 A    来源 R3

Air must still leave the rotor. In the ideal actuator-disk model, the Betz limit is about 59.3%. Real turbines have aerodynamic and downstream losses.

Is a 35% capacity factor the same as 35% efficiency?

讲者 A    来源 R3

No. Capacity factor compares actual energy over time with continuous rated output. Efficiency measures conversion. Our 35% capacity factor is an assumed teaching value.

Why does output not keep increasing with wind speed cubed?

讲者 A    来源 R2, R3

The cubic relation describes available wind power for fixed density and area. Controls limit electrical output at rated power. High winds can cause protective shutdown.

Do all turbines have a gearbox?

讲者 A    来源 R1, R2

No. A direct-drive turbine connects the rotor to the generator without a gearbox. A geared turbine increases shaft speed through a gearbox.

Can a wind plant provide frequency support?

讲者 B    来源 R8, C3

Yes, with suitable controls and within energy and equipment limits. Rotor energy gives a short response. Sustained upward support needs headroom and an available energy source.

问答准备续

How is voltage control different from a synchronous-machine AVR?

讲者 B    来源 R3, C2

An AVR acts on excitation in a synchronous machine. Our full-converter example controls reactive current. Other wind-generator architectures use different arrangements.

Does every wind farm require a battery?

讲者 B    来源 R3, R9

No. The system can use interconnection, flexible generation and demand response as well as storage. The choice depends on the grid need and project economics.

Does 1.3 GW mean constant output from Hornsea 2?

讲者 B    来源 R6, R3

No. It is approximate rated capacity. Actual power and annual energy depend on wind, availability, losses and curtailment.

Is wind power zero-emission?

讲者 B    来源 R5

There is no fuel combustion during generation, but manufacturing and construction cause life-cycle emissions. The chart uses IPCC 2014 literature medians.

How much land does a wind farm occupy?

讲者 B    来源 R7

Separate direct infrastructure footprint from the total project area. Some uses, including agriculture, can continue between turbines where site conditions allow.

问到功率曲线、变流器架构或独立储能时，可分别打开 PPT 第 24、25、26 页。

参考文献

R1  U.S. Department of Energy. How Do Wind Turbines Work?

https://www.energy.gov/cmei/systems/how-do-wind-turbines-work

R2  U.S. Department of Energy. Explore a Wind Turbine

https://www.energy.gov/cmei/systems/explore-wind-turbine-text-version

R3  Burton et al. (2011). Wind Energy Handbook, 2nd edition

https://doi.org/10.1002/9781119992714

R4  IRENA (2025). Renewable Capacity Statistics 2025

https://www.irena.org/Publications/2025/Mar/Renewable-capacity-statistics-2025

R5  IPCC (2014). Annex III, Table A.III.2

https://www.ipcc.ch/site/assets/uploads/2018/02/ipcc_wg3_ar5_annex-iii.pdf

R6  Ørsted. Hornsea 2 Offshore Wind Farm

https://orsted.co.uk/energy-solutions/offshore-wind/our-wind-farms/hornsea2

R7  Denholm et al. (2009). Land-Use Requirements of Modern Wind Power Plants, NREL/TP-6A2-45834

https://www.nrel.gov/docs/fy09osti/45834.pdf

参考文献续

R8  NREL (2014). Active Power Controls from Wind Power, NREL/TP-5D00-60574

https://www1.eere.energy.gov/wind/pdfs/60574.pdf

R9  IEA (2025). World Energy Investment 2025, China

https://www.iea.org/reports/world-energy-investment-2025/china

R10  U.S. Department of Energy. What Is Wind Energy?

https://www.energy.gov/cmei/systems/windexchange/what-wind-energy

C1  Mehrjerdi. Course slides, Introduction, PDF pp. 17–19 and 27

C2  Mehrjerdi. Course slides, Voltage Control, PDF pp. 17–21 and 33

C3  Mehrjerdi. Course slides, Frequency Control, PDF pp. 12–13 and 24–29

课程课件索引沿用原稿的 PDF 页码。

图片来源

PPT 第 2 至 7、13、15 页使用美国能源部网站的照片或示意图。直驱剖面图署名 Josh Bauer / NLR。第 1、18 页照片和第 18 页位置图来自 Ørsted。图片的具体链接已放入各页 PPT 备注。